Sliding Mode Observer (SMO) for PMSM Sensorless Control

## 1. Introduction

Sliding Mode Observer (SMO) is a widely used sensorless control technique for Permanent Magnet Synchronous Motors (PMSMs). Instead of using mechanical position sensors such as encoders or resolvers, SMO estimates the rotor position and speed from the measured stator voltages and currents. The method is highly valued because of its robustness against parameter variations and external disturbances while maintaining relatively low computational complexity.

## 2. PMSM Sensorless FOC Framework

In a sensorless Field Oriented Control (FOC) system, the rotor electrical angle is not directly measured. Instead, a position and speed estimator reconstructs this information using motor electrical signals.

The current feedback measured in the three-phase stator is transformed into the stationary α-β reference frame through the Clarke transformation and then into the rotating d-q frame through the Park transformation. The estimated rotor angle is required for these coordinate transformations.

Figure 1: Overall sensorless PMSM FOC system showing the position and speed estimator

## 3. Principle of Sliding Mode Observation

The fundamental idea of SMO is to reconstruct the PMSM back-EMF components Eα and Eβ in the stationary reference frame. These back-EMF signals contain both rotor position and speed information.

The PMSM model in the α-β frame can be expressed as:

didt=f(u,i,E)

where:

- u = stator voltage

- i = stator current

- E = back-EMF

The observer creates estimated currents i^α and i^β and continuously compares them with the measured currents. With PMSM model, we have

ddtiαiβ=-RsLd-ωeLd-LqLdωeLd-LqLd-RsLdiαiβ+1Lduαuβ-1Ldνανβ

## 4. Observer Structure

The SMO introduces a discontinuous correction term:

v=h⋅sign(i^-i)

where:

- h = sliding gain

- sign(⋅) = sign function

- i^-i = current estimation error

The correction term forces the estimated current trajectory toward the actual current trajectory. When the estimation error approaches zero, the observer state reaches the sliding surface.

νανβ=h⋅signiα-iαh⋅signiβ-iβ

## 5. Sliding Surface Concept

The sliding surface is defined by the current estimation error:

S=i^-i=0

When the observer operates correctly, the state converges toward this surface and remains close to it.

Unlike traditional sliding mode theory diagrams that use error versus error derivative, the article emphasizes that in PMSM SMO analysis the sliding surface should be interpreted in the plane formed by:

- Actual current

- Estimated current

This representation better reflects the physical meaning of observer convergence.

Figure 2. Sliding Surface Representation

## 6. Stability Analysis

The stability of the observer is demonstrated using Lyapunov theory.

A Lyapunov function is selected as:

V=12s2

To guarantee convergence:

V<0

The article derives the necessary condition for selecting the sliding gain h. The gain must be sufficiently large to overcome uncertainties and guarantee that the estimation error converges to zero.

## 7. Sliding Gain Selection

The sliding gain h is one of the most critical design parameters.

Small Gain

If h is too small:

- Current estimation error may not converge.

- Observer stability becomes poor.

- Rotor position estimation becomes inaccurate.

Large Gain

If h is too large:

- Chattering increases.

- Estimated currents oscillate around the actual currents.

- Back-EMF estimation noise increases.

Therefore, the gain should be chosen large enough to guarantee convergence while minimizing oscillation. The article derives this gain analytically instead of relying on trial-and-error tuning.

## 8. Back-EMF Reconstruction and Rotor Position Estimation

Once the observer reaches the sliding surface, the switching signals contain information about the motor back-EMF.

After filtering the switching outputs, the observer obtains:

Eα, Eβ

Rotor electrical angle can then be estimated as:

θe=arctan-EαEβ

and electrical speed can be obtained from the variation of rotor angle over time.

## 9 Simulation and Test

## 10. Advantages of SMO

SMO offers several important advantages:

- Strong robustness against parameter variations.

- Good tolerance to motor resistance and inductance mismatch.

- Low computational burden compared with Extended Kalman Filter (EKF).

- Suitable for embedded motor-control applications.

- Effective for practical PMSM sensorless FOC systems.

## 11. Limitations

Despite its advantages, SMO also has limitations:

- Chattering caused by the switching function.

- Requirement for low-pass filtering of estimated back-EMF.

- Reduced performance at very low speed because back-EMF magnitude becomes small.

- Careful gain tuning is required to balance convergence and noise.

## 12 Conclusion

The Sliding Mode Observer is a robust and computationally efficient sensorless estimation technique for PMSM drives. By constructing an observer in the stationary α-β frame and introducing a switching correction term based on current estimation error, the observer forces estimated currents to converge to measured currents. Once the sliding condition is achieved, the motor back-EMF can be reconstructed and used to estimate rotor position and speed. Proper selection of the sliding gain is crucial, as it determines both convergence performance and chattering behavior. For industrial PMSM sensorless FOC applications, SMO remains one of the most practical and widely adopted observer solutions.